SCA Slag Cement University Exam

While the slag cement university platform is a great way to expand your knowledge on slag cement use, this is not a certification program.  As with all concrete mixtures, trial batches should be performed to verify concrete properties.  Results may vary due to a variety of circumstances, including temperature and mixture components, among other things.  You should consult your slag cement professional for assistance.  Nothing contained herein shall be considered or construed as a warranty or guarantee, either expressed or implied, including any warranty of fitness for a particular purpose.

- Can you replace a portion of Portland Cement with Slag Cement to create a stronger and more durable concrete?

  - Yes

  - no

- T/F: In concrete, slag cement may be proportioned from 20 to 80% depending on the application.

  - True

  - False

- Why is the lower heat evolution characteristic of slag cement in the summer beneficial?

  - because it allows more time for placing and finishing concrete

  - it creates a darker colored concrete

  - the particle size creates a more porous concrete structure

  - Heat evolution is not a factor in the summer

- T/F: The specific gravity of slag cement ranges from 2.85 to 2.94, depending on the source

  - True

  - False

- ______ is defined as the point in time at which penetration resistance reaches specified values.

  - Time of day

  - Time of temperature rise

  - Time of initial set

  - Time of hydration

- Which of the following does not affect time of set?

  - Cementitious material chemistry

  - Cementitious material fineness

  - Cementitious material physics

  - Cementitious material content

- T/F: At temperatures greater than 85 degrees Fahrenheit, concrete containing slag cement can have longer times of set.

  - True

  - False

- ______ are an effective way to control cracking due to shrinkage

  - Saw-cut joints

  - Steel reinforcement

  - High water to cement ratio

  - Poor curing practices

- T/F: The later the concrete can be cut without raveling, the better the chances are that concrete will not crack before saw cutting.

  - True

  - False

- When correctly determining the earliest time for saw cutting, an important consideration is:

  - the type of admixtures in the concrete

  - ambient temperature and environmental conditions at the site

  - the cementitious factor in the mix

  - the types of aggregates used

  - All of the Above

- A concrete mix with quartzite needs to achieve a compressive strength of nearly ______________psi more than a gravel mix before it can be safely saw cut.

  - 400psi

  - 800psi

  - 1200psi

  - 1400psi

- T/F: As a rough guideline, the time to saw cut is delayed approximately 30 minutes for every 20% of slag cement replacing Portland cement.

  - True

  - False

- T/F: Slag cement and Portland cement are stored in a similar manner.

  - True

  - False

- Slag cement must be stored in ____ or ____ to provide protections from moisture and contamination.

  - Bins, silos

  - Train cars, open fields

  - Bags, barrels

  - Bags, train cars

- _____ refers to leveling, smoothing, consolidating and otherwise treating surfaces of fresh, plastic concrete to produce desired surface or appearance.

  - Batching

  - Placing

  - Finishing

  - Curing

- Concrete with lower permeability can generally be achieved by substituting between ______ percent slag cement for Portland cement.

  - 5%-45%

  - 15%-55%

  - 25%-65%

  - 35%-75%

- Slag cement ___ concrete permeability.

  - Increases

  - Reduces

  - Preserves

  - Eliminates

- T/F: When reinforcing steel in concrete corrodes, the product of that corrosion (rust) takes up more volume.

  - True

  - False

- Principle factors that affect the rate and severity of sulfate attack include all of the following EXCEPT:

  - Permeability of the concrete

  - Formation of ettringite

  - C3A content

  - Ca(OH)2 content

- Sulfate attack is a common form of ____.

  - Concrete deterioration

  - Concrete permeability

  - Concrete curing

  - Concrete cracking

- T/F: Slag cement can give a Type I cement the sulfate resisting properties of a Type II cement.

  - True

  - False

- T/F: Slag cement can give a Type I or a Type II cement the sulfate resisting properties of a Type V cement.

  - True

  - False

- Which of the following is not a factor that affects the rate and severity of ASR?

  - The reactivity of the aggregate

  - The availability of alkalies in the concrete

  - The exposure conditions

  - An adequate and proper air void system

- _____ is a chemical reaction between the alkalies in Portland cement and certain types of silica minerals present in some aggregates.

  - Alkali-silica reaction (ASR)

  - Sulfate Attack

  - Creep

  - Greening effect

- T/F: Slag cement can be used to reduce alkali loading in concrete.

  - True

  - False

- In ASR, the reaction product is _____, which absorbs moisture and swells.

  - Hygroscopic gel

  - Calcium hydroxide

  - Sulfates

  - CO2

- Which of the following is a strategy for reducing thermal stress in concrete?

  - Increasing sand in concrete mixture

  - Increasing the Portland cement content

  - Reducing the Portland cement content

  - Increasing the total cementitious content

- Cement hydration generates __.

  - Liquid

  - Sediment

  - Gas/Fumes

  - Heat

- Hardened concrete containing slag cement may show mottled ___ or ___ areas on the surface in the first few days after placement, which goes away within 1-2 weeks.

  - Green; blue green

  - Green; yellow

  - Blue; yellow

  - Yellow; blue green

- T/F: Concrete greening does not affect concrete performance.

  - True

  - False

- The blue-green color, commonly referred to as “greening”, is attributed to a complex reaction of ___________ in slag cement with other compounds in portland cement.

  - Sulfide Sulphur

  - Carbon Dioxide

  - Sulfate Attack

  - Alkali-Silica reaction

- At normally specified replacement levels, concrete made with slag cement will have lower _________ than concrete made with Class F and C fly ash when tested according to ASTM C1202.

  - Strength

  - Color

  - Permeability

  - Finishability

- Slag cement will provide concrete with ______ reflectivity than ordinary portland cement concrete.

  - Lower

  - Higher

- Blended cements incorporating slag and portland cement are specified in _______.

  - ASTM C989

  - ASTM C595

  - ASTM C 1157

  - ASTM C 989

- ASTM C989 Specifies ________ ground granulated blast-furnace slag for use as a cementitious material in concrete and mortar.

  - Roughly

  - Loosely

  - Finely

  - Non

- Slag cement should be used as a pound for pound replacement for a portion of the portland cement in a concrete mixture.

  - True

  - False

- If the specific slag/portland cement mixture is shown to mitigate ASR in accordance with ASTM C1778, low alkali cement is necessary.

  - True

  - False

- ___________ is the non-metalic product, consisting essentially of silicates and aluminosilicates of calcium and other bases, which is developed in a molten condition simultaneously with iron in a blast furnace.

  - Blast furnace

  - Blast furnace slag

  - Hydraulic cement

  - Pozzolan

- Concrete made with slag cement provides higher compressive and flexural strength compared with straight portland cement concrete.

  - True

  - False

- _____ day strengths generally increase as the percentage of slag cement increases, up to about 50% slag cement as a percent of cementitious material.

  - 15

  - 28

  - 30

  - 36

- What product created from portland cement reacting with water is beneficially utilized by slag cement to improve the strength and permeability properties of concrete?

  - Calcium hydrate.

  - Fly ash cement

  - Calcium bicarbonate

  - Magnesium bicarbonate

- Concrete flexural strengths in excess of ______ psi are common with mixes containing slag cement.

  - 10,000

  - 50,000

  - 1,000

  - 5,000

- When used at ______ replacement rates, slag cement will provide _______ heat in mass concrete.

  - High, lower

  - Low, higher

  - High, average

  - Medium, higher

- Engineers and concrete producers who have successfully incorporated slag cement in their HPC mixtures have used slag cement in ranges from ________ percent of cementitious material.

  - 30 to 90

  - 20 to 80

  - 5 to 60

  - 10 to 85

- Slag cement can improve both the plastic and hardened properties of precast and prestressed concrete.

  - True

  - False

- Slag cement _________ concrete permeability, which in turn increases resistance to chloride intrusion.

  - Decreases

  - Reduces

  - Eliminates

  - Increases

- Slag cement improves the workability of concrete due to its _______ and ________ structure.

  - Fine grind, glassy

  - Coarse grind, round

  - Fine grind, round

  - Coarse grind, flat

- Slag cement is not an ideal material for concrete pipe.

  - True

  - False

- Elevated curing temperatures _________  early age strength development in mixtures containing slag cement..

  - Deactivate

  - Activate

  - Neutralize

- Slag cement ________ surface texture and density when used in concrete block mixtures.

  - Decreases

  - Doesn’t Affect

  - Improves

- At ______ days, strengths are superior when concrete pipe is manufactured with slag cement.

  - 50

  - 15

  - 28

  - 35

- Block manufactured with slag cement will have a ______ finished color.

  - Darker

  - Lighter

  - Red

  - Green

- Slag cement combines with _____________ and ___________ in portland cement to reduce efflorescence.

  - Calcium hydroxide & soluble salts

  - Nitrogen and Hydrogen

  - Calcium and Nitrogen

  - Calcium hydroxide and magnesium

- What is a benefit of slag cement in residential construction?

  - Improved workability

  - lighter color

  - smoother surface

  - All of the above

- Slag cement is advantageous in decorative concrete because of its lighter color?

  - True

  - False

- Ternary concrete mixtures include ______ different cementitious materials.

  - 5

  - 2

  - 3

  - 6

- Ternary blends have performed for many years in severe conditions with no apparent loss of durability.

  - False

  - True

- Mixture proportions for ternary blends are dependent on all of the following except:

  - Intended application of the concrete

  - The end user

  - construction requirements

  - seasonal conditions.

- Ternary mixtures can be used in ______ concrete applications.

  - Most

  - Few

  - Any

- Utilization of slag cement in concrete _______ the burden on landfills.

  - Increases

  - Lessens

  - Eliminates

  - Stagnates

- In high performance and mass concrete applications slag cement can be used as a substitute up to ______ percent.

  - 50%

  - 25%

  - 80%

  - 75%

- Slag cement requires nearly _____ percent less energy to produce than and equivalent amount of Portland cement.

  - 50%

  - 75%

  - 25%

  - 90%

- Because slag cement has a lighter color it reduces the “________” effect.

  - Heat island

  - Swamp mass

  - Cold land

- What is not a reason to add supplementary cementitious materials (SCMs) (Slag Cement, Fly Ash, etc.) to a concrete mix design?

- SCMs help form more Calcium-Silica Hydrates (C-S-H).

- SCMs will increase the amount of Portland cement in the mix.

- SCMs remove excess alkalis from pore water.

- SCM’s reduce the environmental footprint of concrete.

- How is slag cement manufactured?

- Slag cement is formed when molten slag, diverted from a lead blast furnace, is rapidly heated producing glassy granules ground to cement fineness.

- Slag cement is formed when molten slag, diverted from an iron blast furnace, is rapidly quenched producing glassy granules ground to cement fineness.

- Slag cement is formed when molten slag, diverted from a lead blast furnace, is rapidly quenched producing glassy granules ground to cement fineness.

- Slag cement is formed when molten slag, diverted from an iron blast furnace, is rapidly heated producing glassy granules ground to cement fineness.

- Slag cement particles have a _________ structure/shape.

- Round

- Square

- Glassy/angular

- Flat

- Slag cement composition is closer to Portland cement than other scms.

- True

- False

- The following are standard specifications or guides on slag cement use in concrete, EXCEPT _________.

- ASTM C989

- ACI 480

- AASHTO M302

- ACI 233R-17

- The reaction of cement and water will produce two primary hydration products:

- Calcium Silica Hydrate (CSH) and Calcium Hydroxide (CaOH).

- Tricalcium Aluminate (C3A) and Dicalcium Silicate (C2S)

- Calcium Sulfate (CaSO4) and Silicon Dioxide (SiO2)

- Tricalcium Silicate (C3S) and Calcium Hydroxide (CaOH)

- Slag cements time of set increases versus reference Portland cement by ½ to 1 hour at 73 degrees Fahrenheit (23 degrees Celsius). There is little change above 85 degrees Fahrenheit.

- True

- False

- The following statements are true about slag cements effect on concrete strength gain, except:

- Slag cement leads to greater compressive and flexural strength at later ages (7 days and beyond).

- Slag cement has no affect on strength gain.

- Slag cement use leads to less strength at early ages (1 to 7 days).

- Optimum dosage of slag cement for 28-day strength is dependent on application, time of year (temperature), and replacement percentage.

- Which of the following is not a benefit of slag cement use in concrete durability?

- ASR mitigation

- Increases concrete permeability

- Increased sulfate resistance

- Reduces concrete permeability

- Slag cement ________ the total alkalis in the system, _________ the alkali-silica ratio and the potential for alkali silica reaction (ASR).

- Increases, increases

- Reduces, reduces

- Increases, reduces

- Reduces, increases

- The use of slag cement is an extremely effective way of reducing risk of sulfate attack for the following reasons except:

- Reduces the total amount of C3A in the system.

- Reduces concrete permeability, making it harder for sulfates to penetrate into concrete.

- Concrete made with slag cement increases permeability.

- Reacts with Ca(OH)2 to form additional calcium silicate hydrate reducing the Ca(OH)2 in the system.

- The following are methods to measure concrete permeability, except:

- Rapid chloride permeability test – ASTM C1202 “Electrical Indication of Concrete's Ability to Resist Chloride Ion Penetration”

- Penetration test – AASHTO T 358-17 “Standard Test for Surface Resistivity Indication of Concrete’s Ability to Resist Chloride Ion Penetration”

- NEED A DUMMY ANSWER – TESTING SUGGESTION?

- Formation Factor – The Standard Method of Test for Electrical Resistivity of a Concrete Cylinder Tested in a Uniaxial Resistance Test (AASHTO TP119)

- Which of the following are applications of slag cement in ready mixed concrete?

- Self Consolidating Concrete (SCC)

- High Performance Concrete

- Roller Compacted Concrete

- Controlled Low Strength Material

- All of the Above

- Slag cement use leads to lower heat of hydration in mass concrete by __________% compared to portland cement concrete.

- 5-10%

- 15-50%

- 20-60%

- 75=95%

- Concrete made with slag cement tends to be lighter and brighter in color with more reflectivity than mix designs with only Portland cement.

- True

- False

- What does the Life Cycle Assessment Calculator for Slag Cement do?

- Calculates cost estimates of slag cement mix design options.

- Shows comparisons of environmental impact of various mix designs with slag cement at various percentage replacement rates, based on industry wide Environmental Product Declaration information.

- Identifies recommended replacement rates for slag cement based on application.

- Shows comparisons of strength characteristics of various mix designs with slag cement at various replacement rates, based on industry wide data.

- The following are environmental benefits of slag cement use in concrete except:

- Less energy consumption is associated with production.

- It is a recycled material.

- Using slag cement increases the amount of Portland cement used in a mix design.

- Pavements have greater reflectivity from lighter color of concrete.

Slag Cement University Exam Questions Material Reference

SC- 2: Concrete Proportioning – 2-4

SC- 3: Concrete Time of Set – 4-7

SC- 4: Saw Cutting Joints – 8-12

SC- 5: Producing and Placing Slag Cement Concrete – 13-18

SC- 6: Reducing Permeability – 20, 21, 22

SC- 7: Mitigating Sulfate Attack – 19-22

SC- 8: Mitigating Risk of ASR – 23-26

SC- 9: Reducing Thermal Stress in Concrete – 27, 28

SC- 10: Greening – 29-31

SC- 11: Slag Cement and Fly Ash – 32

SC- 12: Terminology and Specifications, SC- 13: Suggested Specification Provision for Slag Cement in Concrete – 33-38

SC- 14: Compressive and Flexural Strength – 39-42

SC- 15: Slag Cement in High Performance Concrete – 43,44

SC- 16: Producing Precast and Prestressed Concrete with Slag Cement, SC- 17: Producing Concrete Pipe with Slag Cement, SC- 18. Producing concrete Block with Slag Cement – 45- 52

SC- 19. Residential Concrete – 53-55

SC -20. Ternary Concrete Mixtures with Slag Cement – 56-59

SC-22. Slag Cement and the Environment – 60-63

Slag Cement 101 Presentation/Jay Whitt Presentation, 1, 64-80